1. Overview & Key Characteristics

- Definition: I2C stands for Inter-Integrated Circuit. It is a highly popular, byte-oriented protocol used widely in embedded systems.

- Communication Type: Serial, synchronous, and half-duplex (data can only travel in one direction at a time).

- Network Support: It supports multiple masters and multiple slaves connected to the same bus.

- Distance: Designed for short-distance, intra-board communication (less than 1 meter or 50 centimeters).

- Speeds:

  - Standard speeds: 100 kbps to 400 kbps.

  - High/Ultra-high speeds: Up to 1 Mbps or 3.5 Mbps.

2. Hardware & Wiring

- Two-Wire Interface: The protocol uses only two wires for communication:

  - SDA (Serial Data): The line used to transmit data.

  - SCL (Serial Clock): The line used for the clock signal, which is always generated and controlled by the master.

- Circuit Design: Both SDA and SCL are open-drain lines and must be connected to a VCC supply using pull-up resistors to control current and communication speed.

3. Addressing Mechanism

- I2C relies on address-based communication, meaning two masters cannot communicate with each other; communication must occur between a master and a slave.

- 7-bit Addressing: Allows connection of up to 128 (2^7) distinct slave devices.

- 10-bit Addressing: Allows connection of up to 1,024 distinct slave devices.

4. I2C Frame Structure

Communication happens in frames, consisting of the following sequential fields:

- Start Condition: Initiated by the master to begin communication. The SDA line is pulled low before the SCL clock line goes low.

- Slave Address: The master sends the 7-bit or 10-bit address of the specific slave it wants to target.

- Read/Write (R/W) Bit: A single bit indicating the operation. 0 means Write (master sends data), and 1 means Read (master receives data).

- Acknowledgement (ACK): After receiving the 8 bits (address + R/W bit), the targeted slave responds with an active-low bit (0) to confirm its presence. ACKs are also sent after every byte of data transmitted successfully.

- Data: Data is transmitted sequentially, one byte (8 bits) at a time.

- Stop Condition: Initiated by the master to halt communication and release the bus. The SCL line goes high before the SDA line goes high.

5. Key Operational Concepts

- Arbitration: A process used to resolve conflicts when multiple masters try to start communication at the exact same time.

  - The bus compares the transmitted bits starting from the Most Significant Bit (MSB).

  - In embedded communication, 0 is dominant (high priority) and 1 is recessive (low priority).

  - If Master A sends a 1 while Master B sends a 0, Master A loses arbitration, exits the process, and Master B takes control of the bus.

- Clock Stretching: A feature that allows a slower slave device to match the communication speed of a faster master.

  - Before sending an ACK (typically on the 9th clock pulse), the slave will actively pull the SCL line low.

  - This suspends data transmission temporarily until the slave is ready to proceed and releases the clock line.

6. Operating Modes

A microcontroller using I2C can be configured into four different modes:

- Master Transmitter Mode: Master sends data to a slave.

- Master Receiver Mode: Master receives data from a slave.

- Slave Transmitter Mode: Slave sends data to the master.

- Slave Receiver Mode: Slave receives data from the master.

7. Advantages & Disadvantages

Advantages:

- Requires fewer wires (only two) to connect multiple devices.

- Extremely flexible with speed (Standard, High, Ultra-high) and addressing (7-bit and 10-bit).

- Built-in mechanism to confirm successful transmissions (ACK/NACK).

- Widely adopted; almost all microcontrollers have built-in I2C support.

- Allows slow devices to keep up using clock stretching.

Disadvantages:

- It is a low-speed protocol compared to alternatives like SPI and CAN.

- Distance is strictly limited to short, intra-board connections.

- Half-duplex communication limits simultaneous sending and receiving.

- Requires external hardware (pull-up resistors).

- Possibility of address conflicts if multiple slaves share the same address.

- Heavy data overhead because data must be transmitted strictly byte-by-byte.

- Lacks formal error-checking mechanisms like checksums