### Introduction (Page 3) – Explanation

The I²C-bus (Inter-Integrated Circuit bus) is a widely used communication standard that allows different integrated circuits (ICs) within an electronic system to communicate with each other using a simple two-wire interface. It has become a global standard and is implemented in more than 1000 ICs produced by over 50 companies.

I2C_UM10204

The I²C bus is commonly used in many control and communication architectures such as System Management Bus (SMBus), Power Management Bus (PMBus), Intelligent Platform Management Interface (IPMI), Display Data Channel (DDC), and ATCA systems. These systems rely on the I²C bus for exchanging control and data information between components.

I2C_UM10204

The main purpose of the document is to help device designers and system engineers understand the working of the I²C bus and implement it in practical applications. It explains important aspects such as data transfer mechanisms, handshaking methods, bus arbitration, operating modes, and electrical/timing specifications required for proper communication between devices.

I2C_UM10204

The I²C bus was developed by Philips Semiconductors (now NXP Semiconductors) to simplify communication between ICs in electronic systems. Instead of using many connection lines, the I²C bus uses only two lines:

- SDA (Serial Data Line) – carries the data

- SCL (Serial Clock Line) – carries the clock signal

All devices connected to the bus contain an on-chip interface, allowing them to communicate directly with each other through these two lines. This approach reduces wiring complexity and solves many interfacing problems in digital control circuits.

I2C_UM10204

The document also mentions the I3C protocol, introduced later by the MIPI Alliance in 2017, which is compatible with I²C but provides higher speed, lower power consumption, and improved performance.

## Chapter 2 – I²C Bus Features (Explanation)

Chapter 2 of the document explains the important features of the I²C bus, which make it a simple and efficient communication system for connecting multiple ICs such as microcontrollers, memory devices, sensors, and display drivers.

I2C_UM10204

### 1. Two-Wire Communication

The I²C bus uses only two lines for communication:

- SDA (Serial Data Line) – used to transfer data

- SCL (Serial Clock Line) – used to synchronize data transfer

Because only two wires are required, the circuit becomes simple and requires fewer connections.

I2C_UM10204

### 2. Unique Device Addressing

Each device connected to the I²C bus has a unique address.

- The controller sends the address of the target device before sending data.

- Only the device with the matching address responds.

This allows many devices to share the same bus without conflict.

I2C_UM10204

### 3. Multi-Controller Capability

The I²C bus supports multiple controllers (masters).

- More than one controller can attempt to communicate on the bus.

- Arbitration and collision detection ensure that only one controller completes the transfer.

This prevents data corruption during simultaneous communication.

I2C_UM10204

### 4. Bidirectional Data Transfer

Data transfer on the I²C bus is bidirectional.

- Devices can act as transmitters or receivers depending on the communication direction.

- Data is transferred in 8-bit format (byte-oriented communication).

I2C_UM10204

### 5. Different Data Transfer Speeds

I²C supports multiple speed modes:

| Mode | Data Rate |
| --- | --- |
| Standard mode | up to 100 kbit/s |
| Fast mode | up to 400 kbit/s |
| Fast-mode Plus | up to 1 Mbit/s |
| High-speed mode | up to 3.4 Mbit/s |
| Ultra Fast-mode | up to 5 Mbit/s (unidirectional) |

These modes allow I²C to be used in many types of electronic systems.

I2C_UM10204

### 6. Noise Protection

The bus includes on-chip filtering that removes unwanted spikes or noise on the data line.
This helps maintain reliable communication between devices.

I2C_UM10204

### 7. Flexible Number of Devices

The number of devices connected to the bus mainly depends on the maximum bus capacitance.
Under certain conditions, many ICs can be connected to the same bus.

# 2.1 Designer Benefits

These benefits help system designers while designing electronic circuits.

- Faster system design
Functional blocks in the block diagram correspond directly to available ICs, allowing designers to quickly move from a block diagram to the final circuit design.

- No need for additional interfaces
I²C devices already include the required bus interface on the chip, so external interfacing circuits are not required.

- Software-based control
Addressing and data transfer are handled through software, making the system flexible and easy to modify.

- Reusable components
The same IC types can often be used in different applications, which simplifies design.

- Easy system modification
ICs can be added or removed from the bus without affecting other devices.

- Simple debugging
Fault detection and debugging are easier because the bus structure is simple.

- Reduced software development time
Software modules developed for one system can be reused in others.

- Suitable for portable devices
I²C ICs typically have low power consumption, wide supply voltage range, high noise immunity, and wide operating temperature range.

I2C_UM10204

# 2.2 Manufacturer Benefits

I²C also provides advantages to equipment manufacturers.

- Reduced number of connections
Since only two bus lines are required, the number of interconnections between ICs is minimized.

- Fewer IC pins required
Because communication uses only two lines, devices require fewer pins.

- Smaller and cheaper PCBs
Fewer connections and pins lead to smaller printed circuit boards and reduced cost.

- No need for additional logic circuits
The integrated I²C protocol removes the need for address decoders or other glue logic.

- Flexible product development
Manufacturers can create different versions of equipment simply by adding or removing ICs from the bus.

I2C_UM10204

# 2.3 IC Designer Benefits

These benefits apply to designers of microcontrollers and integrated circuits.

- Reduced pin usage
The I²C protocol allows many peripherals to connect using only two pins, saving valuable microcontroller pins.

- Easy peripheral connection
A wide range of peripheral devices such as memory, sensors, and display drivers can be connected easily.

- Better market acceptance
Microcontrollers with built-in I²C interfaces are more attractive because they support many existing I²C-compatible devices.

I2C_UM10204

# 3.1 I²C-Bus Protocol (Standard-mode, Fast-mode, Fast-mode Plus)

The I²C bus uses two lines:

- SDA (Serial Data Line) – transfers data

- SCL (Serial Clock Line) – synchronizes communication

A controller initiates communication and generates the clock, while the addressed device becomes the target. Data is transferred in 8-bit bytes between devices.

I2C_UM10204

# 3.1.1 SDA and SCL Signals

SDA and SCL are bidirectional lines connected to a power supply through pull-up resistors.

- When the bus is idle, both lines remain HIGH.

- Devices connected to the bus use open-drain or open-collector outputs, which allows multiple devices to share the same lines.

- Data transfer speed depends on the operating mode:

  - Standard mode – up to 100 kbit/s

  - Fast mode – up to 400 kbit/s

  - Fast-mode Plus – up to 1 Mbit/s

  - High-speed mode – up to 3.4 Mbit/s

The number of devices connected to the bus mainly depends on the bus capacitance.

I2C_UM10204

# 3.1.2 Bit Transfer

Data is transferred bit by bit over the SDA line while the SCL line provides the clock.

Important rules:

- Data on SDA must remain stable during the HIGH period of SCL.

- Data can change only when SCL is LOW.

- Each clock pulse transfers one bit of data.

This ensures correct synchronization between transmitting and receiving devices.

# 3.1.3 START and STOP Conditions

Communication on the I²C bus always begins with a START condition and ends with a STOP condition.

- START condition (S):
Occurs when the SDA line changes from HIGH to LOW while SCL is HIGH.

- STOP condition (P):
Occurs when SDA changes from LOW to HIGH while SCL is HIGH.

After a START condition, the bus becomes busy, and it becomes free again after the STOP condition.

# 3.1.4 Byte Format

Data is transmitted in 8-bit bytes.

- Each byte contains 8 data bits.

- After sending 8 bits, a ninth clock pulse is used for the acknowledge signal.

- The number of bytes in a transfer is not limited.

# 3.1.5 Acknowledge (ACK) and Not-Acknowledge (NACK)

After every byte transfer, the receiver sends an acknowledge signal.

ACK (Acknowledge):

- The receiver pulls the SDA line LOW during the ninth clock pulse.

- This indicates that the byte was successfully received.

NACK (Not-Acknowledge):

- SDA remains HIGH during the ninth clock pulse.

NACK can occur in several situations:

- No device with the transmitted address exists.

- The receiver is busy and cannot communicate.

- The receiver does not understand the received data.

- The receiver cannot accept more data.

- The controller wants to end the communication.

I2C_UM10204

# 3.1.6 Clock Synchronization

In systems with multiple controllers, clocks must be synchronized.

- All controllers share the SCL line using wired-AND connection.

- If one controller produces a longer LOW period, the SCL line stays LOW until that controller releases it.

- This ensures all controllers operate with a synchronized clock signal.

I2C_UM10204

# 3.1.7 Arbitration

Arbitration is used when multiple controllers attempt to use the bus simultaneously.

- Arbitration occurs bit by bit during communication.

- While SCL is HIGH, each controller checks whether the SDA level matches the bit it transmitted.

- If a controller sends HIGH but reads LOW, it loses arbitration and stops transmitting.

The winning controller continues the communication without data corruption.

I2C_UM10204

# 3.1.8 Clock Stretching

Clock stretching allows slower devices to delay communication.

- A target device can hold the SCL line LOW after receiving a byte.

- This forces the controller to wait until the target device is ready for the next byte.

Clock stretching acts as a handshaking mechanism between devices.

# 3.1.9 Target Address and R/W Bit

After the START condition, the controller sends a target address.

- Each device has a unique address.

- The address is followed by a Read/Write (R/W) bit:

  - 0 → Write operation

  - 1 → Read operation

This identifies the device and the direction of communication.

# 3.1.10 START Byte

Some microcontrollers monitor the bus through software rather than hardware.

To help them detect communication easily, a START byte sequence can be used:

- START condition

- START byte (00000001)

- Acknowledge clock

- Repeated START condition

This allows slower devices to synchronize with the bus.

I2C_UM10204

# 3.1.11 Bus Clear

If the bus becomes stuck (for example, SDA held LOW):

- The controller sends nine clock pulses on SCL.

- The device holding the line LOW should release it.

- If not, the system must be reset or power-cycled.

# 3.1.12 Device ID

The Device ID field is an optional 24-bit identification value.

It contains:

- 12 bits – manufacturer ID

- 9 bits – part identification

- 3 bits – device revision

This helps identify the exact device connected to the bus.

## 3.2 Ultra Fast-mode (UFm) I²C-Bus

Section 3.2 explains the Ultra Fast-mode (UFm) of the I²C bus. This mode is designed for applications that require very high-speed data transfer. Unlike normal I²C communication, UFm is unidirectional, meaning data flows only from the controller to the target device. It can achieve speeds of up to 5 Mbit/s.

I2C_UM10204

UFm uses two signals similar to normal I²C:

- USDA (Ultra Serial Data Line) – used to transmit data

- USCL (Ultra Serial Clock Line) – used to provide clock timing

These lines use push-pull outputs instead of open-drain outputs to achieve faster signal transitions.

# 3.2.1 USDA and USCL Signals

The UFm I²C bus uses two communication lines:

- USDA – carries serial data.

- USCL – carries the clock signal generated by the controller.

Important characteristics:

- Both lines are unidirectional.

- They use push-pull drivers, allowing faster switching.

- Only one controller is allowed on the bus to avoid signal conflicts.

Because of the push-pull configuration, arbitration and clock stretching are not supported in UFm mode.

# 3.2.2 USDA and USCL Logic Levels

Different technologies such as CMOS, NMOS, and bipolar devices may be connected to the I²C bus. Therefore, the logic levels are defined relative to the supply voltage VDD.

Typical logic levels:

- LOW level (VIL) ≈ 0.3 × VDD

- HIGH level (VIH) ≈ 0.7 × VDD

These levels ensure reliable communication even when devices operate at different supply voltages.

I2C_UM10204

# 3.2.3 Data Validity

In UFm communication, data must follow strict timing rules.

- The data on the USDA line must remain stable when the clock (USCL) is HIGH.

- Data changes are allowed only when the clock is LOW.

- Each clock pulse transfers one bit of data.

This rule ensures that the receiver reads the correct data during each clock cycle.

I2C_UM10204

# 3.2.4 START and STOP Conditions

Communication on the UFm I²C bus begins and ends with special conditions.

### START Condition (S)

A START condition occurs when:

- The USDA line changes from HIGH to LOW

- While the USCL line is HIGH

This indicates the beginning of data transmission.

### STOP Condition (P)

A STOP condition occurs when:

- The USDA line changes from LOW to HIGH

- While the USCL line is HIGH

This indicates the end of communication.

After the START condition, the bus becomes busy until the STOP condition occurs.

I2C_UM10204

# 3.2.5 Byte Format

In Ultra Fast-mode:

- Data is transmitted in 8-bit bytes.

- Each byte consists of 8 data bits sent sequentially on the USDA line.

- The number of bytes transmitted in one communication sequence is not restricted.

Unlike standard I²C modes, UFm typically does not include acknowledge signals because communication is unidirectional and optimized for high speed.