Chapter 1: Introduction (Cortex-M0 Processor)

1.1 About the Processor

The Cortex-M0 processor is a 32-bit ARM processor designed for microcontrollers and embedded systems. It is optimized to provide low power consumption, small silicon area, and efficient performance.

Key characteristics:

- Very low gate count processor.

- Energy-efficient, suitable for battery-powered devices.

- Designed for deeply embedded applications.

- Used in devices like IoT systems, sensors, automotive electronics, and consumer electronics.

Because of its small size and efficiency, it is widely used in cost-sensitive microcontroller systems.

DDI0432C_cortex_m0_r0p0_trm

Main Purpose

The processor is designed to:

- Execute embedded software efficiently

- Provide deterministic interrupt handling

- Reduce system cost and power consumption

1.2 Features of Cortex-M0

The Cortex-M0 processor includes several important features that improve performance, power efficiency, and development simplicity.

1. Tight Integration of System Peripherals

The processor integrates closely with system components like:

- interrupt controllers

- timers

- memory interfaces

Benefits:

- Reduces hardware area

- Simplifies system design

- Lowers development cost

2. Thumb Instruction Set

The processor uses the Thumb instruction set.

Characteristics:

- Mostly 16-bit instructions

- High code density

- Efficient memory usage

Advantages:

- Smaller programs

- Faster execution in embedded systems

- Reduced memory requirement

3. Power Control Optimization

The processor includes features that allow power management of system components.

Examples:

- Sleep modes

- Clock control

- Low-power states

This helps extend battery life in portable devices.

4. Integrated Sleep Modes

Cortex-M0 supports sleep modes to reduce power consumption.

When the processor enters sleep mode:

- The CPU stops executing instructions

- Power consumption drops significantly

- Processor wakes when an interrupt occurs

This is important for:

- IoT devices

- Battery-powered systems

- Low-energy embedded applications

5. Fast Code Execution

Efficient execution allows:

- Lower clock frequency operation

- Longer sleep periods

Result:

- Lower energy consumption

- Improved system efficiency

6. Hardware Multiplier

The processor includes a 32-bit hardware multiplier.

This allows fast execution of multiplication operations, which is important for:

- signal processing

- control algorithms

- mathematical computations

7. Deterministic Interrupt Handling

The processor includes a high-performance interrupt system.

Advantages:

- Predictable response time

- Suitable for real-time systems

- Supports time-critical applications

Examples:

- motor control

- industrial automation

- robotics

8. Serial Wire Debug (SWD)

Cortex-M0 supports Serial Wire Debug, which allows debugging with fewer pins.

Benefits:

- Simplifies hardware design

- Reduces pin count

- Enables easier debugging of embedded programs

1.3 Interfaces

The processor communicates with external systems using two major interfaces.

1. External AHB-Lite Interface

Cortex-M0 uses the AMBA AHB-Lite bus for system communication.

AHB-Lite stands for:
Advanced High-performance Bus Lite

Functions:

- Connects processor to memory

- Connects processor to peripherals

- Handles data transfers

Advantages:

- High performance

- Simple bus architecture

- Easy integration with SoC systems

2. Debug Access Port (DAP)

The processor includes a Debug Access Port.

Purpose:

- Provides debugging access to the processor

- Allows external debugger connection

Functions:

- Read/write memory

- Inspect registers

- Control program execution

Debug connections can use:

- Serial Wire Debug (SWD)

- JTAG interface

DDI0432C_cortex_m0_r0p0_trm

1.4 Configurable Options

Cortex-M0 allows several configuration options during system implementation.

These options allow designers to optimize:

- performance

- cost

- power consumption

1. Number of External Interrupts

Possible options:

- 1

- 2

- 4

- 8

- 16

- 24

- 32 interrupts

More interrupts allow the system to handle more external events.

2. Data Endianness

The processor supports two memory formats:

Little-Endian

- Least significant byte stored first

- Most commonly used

Big-Endian

- Most significant byte stored first

3. SysTick Timer

The system may include a SysTick timer.

Purpose:

- Generates periodic interrupts

- Used for operating systems and scheduling

It can be:

- Present

- Absent

4. Watchpoint Comparators

Used for debugging memory accesses.

Possible values:

- 0

- 1

- 2 watchpoints

5. Breakpoint Comparators

Used to stop program execution during debugging.

Possible values:

- 0 to 4 breakpoints

6. Halting Debug Support

Determines whether debugging features are available.

Options:

- Present

- Absent

7. Multiplier Implementation

The hardware multiplier can be implemented in two ways:

- Fast Multiplier

  - Single cycle operation

  - Higher performance

- Iterative Multiplier

  - Takes up to 32 cycles

  - Uses less hardware area

Important note:
Even if multiplication is in progress, the processor can interrupt it to handle interrupts quickly.

DDI0432C_cortex_m0_r0p0_trm

1.5 Product Documentation, Design Flow and Architecture

This section explains how the Cortex-M0 processor is used during system design and development.

1. Documentation Types

Technical Reference Manual (TRM)

The TRM explains:

- processor functionality

- architecture

- registers

- configuration options

It is used by:

- software developers

- system designers

Integration and Implementation Manual (IIM)

This manual describes:

- how to configure the processor hardware

- integration into System-on-Chip (SoC)

- configuration of RTL (Register Transfer Level)

It is mainly used by hardware engineers.

Design Flow of Cortex-M0

The processor goes through three stages before being used in a product.

1. Implementation

The processor RTL is synthesized to create hardware.

Tasks include:

- selecting configuration options

- generating the macrocell design

2. Integration

The processor is integrated into a System-on-Chip (SoC).

This involves connecting:

- memory

- peripherals

- bus systems

3. Programming

Software developers write programs that:

- configure processor registers

- initialize hardware

- run application software

Architecture and Protocols

Cortex-M0 follows several ARM standards.

1. ARM Architecture

The processor implements the ARMv6-M architecture.

This defines:

- instruction set

- register structure

- exception handling

2. AMBA Bus Architecture

The processor uses AMBA-3 AHB-Lite for system communication.

This bus connects:

- processor

- memory

- peripherals

3. Debug Access Port Architecture

Debugging follows the ARM Debug Interface specification.

This enables:

- debugging tools

- breakpoints

- system inspection

1.6 Product Revisions

Processor versions are identified using revision numbers.

Format:

rnpn

Where:

- rn → major revision

- pn → minor revision

Example:

r0p0

Meaning:

- major revision = 0

- minor revision = 0

The manual currently describes the first release r0p0 of the Cortex-M0 processor.

Chapter 1: Introduction (Cortex-M0 Processor)

1.1 About the Processor

The Cortex-M0 processor is a 32-bit ARM processor designed for microcontrollers and embedded systems. It is optimized to provide low power consumption, small silicon area, and efficient performance.

Key characteristics:

- Very low gate count processor.

- Energy-efficient, suitable for battery-powered devices.

- Designed for deeply embedded applications.

- Used in devices like IoT systems, sensors, automotive electronics, and consumer electronics.

Because of its small size and efficiency, it is widely used in cost-sensitive microcontroller systems.

DDI0432C_cortex_m0_r0p0_trm

Main Purpose

The processor is designed to:

- Execute embedded software efficiently

- Provide deterministic interrupt handling

- Reduce system cost and power consumption

1.2 Features of Cortex-M0

The Cortex-M0 processor includes several important features that improve performance, power efficiency, and development simplicity.

1. Tight Integration of System Peripherals

The processor integrates closely with system components like:

- interrupt controllers

- timers

- memory interfaces

Benefits:

- Reduces hardware area

- Simplifies system design

- Lowers development cost

2. Thumb Instruction Set

The processor uses the Thumb instruction set.

Characteristics:

- Mostly 16-bit instructions

- High code density

- Efficient memory usage

Advantages:

- Smaller programs

- Faster execution in embedded systems

- Reduced memory requirement

3. Power Control Optimization

The processor includes features that allow power management of system components.

Examples:

- Sleep modes

- Clock control

- Low-power states

This helps extend battery life in portable devices.

4. Integrated Sleep Modes

Cortex-M0 supports sleep modes to reduce power consumption.

When the processor enters sleep mode:

- The CPU stops executing instructions

- Power consumption drops significantly

- Processor wakes when an interrupt occurs

This is important for:

- IoT devices

- Battery-powered systems

- Low-energy embedded applications

5. Fast Code Execution

Efficient execution allows:

- Lower clock frequency operation

- Longer sleep periods

Result:

- Lower energy consumption

- Improved system efficiency

6. Hardware Multiplier

The processor includes a 32-bit hardware multiplier.

This allows fast execution of multiplication operations, which is important for:

- signal processing

- control algorithms

- mathematical computations

7. Deterministic Interrupt Handling

The processor includes a high-performance interrupt system.

Advantages:

- Predictable response time

- Suitable for real-time systems

- Supports time-critical applications

Examples:

- motor control

- industrial automation

- robotics

8. Serial Wire Debug (SWD)

Cortex-M0 supports Serial Wire Debug, which allows debugging with fewer pins.

Benefits:

- Simplifies hardware design

- Reduces pin count

- Enables easier debugging of embedded programs

1.3 Interfaces

The processor communicates with external systems using two major interfaces.

1. External AHB-Lite Interface

Cortex-M0 uses the AMBA AHB-Lite bus for system communication.

AHB-Lite stands for:
Advanced High-performance Bus Lite

Functions:

- Connects processor to memory

- Connects processor to peripherals

- Handles data transfers

Advantages:

- High performance

- Simple bus architecture

- Easy integration with SoC systems

2. Debug Access Port (DAP)

The processor includes a Debug Access Port.

Purpose:

- Provides debugging access to the processor

- Allows external debugger connection

Functions:

- Read/write memory

- Inspect registers

- Control program execution

Debug connections can use:

- Serial Wire Debug (SWD)

- JTAG interface

DDI0432C_cortex_m0_r0p0_trm

1.4 Configurable Options

Cortex-M0 allows several configuration options during system implementation.

These options allow designers to optimize:

- performance

- cost

- power consumption

1. Number of External Interrupts

Possible options:

- 1

- 2

- 4

- 8

- 16

- 24

- 32 interrupts

More interrupts allow the system to handle more external events.

2. Data Endianness

The processor supports two memory formats:

Little-Endian

- Least significant byte stored first

- Most commonly used

Big-Endian

- Most significant byte stored first

3. SysTick Timer

The system may include a SysTick timer.

Purpose:

- Generates periodic interrupts

- Used for operating systems and scheduling

It can be:

- Present

- Absent

4. Watchpoint Comparators

Used for debugging memory accesses.

Possible values:

- 0

- 1

- 2 watchpoints

5. Breakpoint Comparators

Used to stop program execution during debugging.

Possible values:

- 0 to 4 breakpoints

6. Halting Debug Support

Determines whether debugging features are available.

Options:

- Present

- Absent

7. Multiplier Implementation

The hardware multiplier can be implemented in two ways:

- Fast Multiplier

  - Single cycle operation

  - Higher performance

- Iterative Multiplier

  - Takes up to 32 cycles

  - Uses less hardware area

Important note:
Even if multiplication is in progress, the processor can interrupt it to handle interrupts quickly.

DDI0432C_cortex_m0_r0p0_trm

1.5 Product Documentation, Design Flow and Architecture

This section explains how the Cortex-M0 processor is used during system design and development.

1. Documentation Types

Technical Reference Manual (TRM)

The TRM explains:

- processor functionality

- architecture

- registers

- configuration options

It is used by:

- software developers

- system designers

Integration and Implementation Manual (IIM)

This manual describes:

- how to configure the processor hardware

- integration into System-on-Chip (SoC)

- configuration of RTL (Register Transfer Level)

It is mainly used by hardware engineers.

Design Flow of Cortex-M0

The processor goes through three stages before being used in a product.

1. Implementation

The processor RTL is synthesized to create hardware.

Tasks include:

- selecting configuration options

- generating the macrocell design

2. Integration

The processor is integrated into a System-on-Chip (SoC).

This involves connecting:

- memory

- peripherals

- bus systems

3. Programming

Software developers write programs that:

- configure processor registers

- initialize hardware

- run application software

Architecture and Protocols

Cortex-M0 follows several ARM standards.

1. ARM Architecture

The processor implements the ARMv6-M architecture.

This defines:

- instruction set

- register structure

- exception handling

2. AMBA Bus Architecture

The processor uses AMBA-3 AHB-Lite for system communication.

This bus connects:

- processor

- memory

- peripherals

3. Debug Access Port Architecture

Debugging follows the ARM Debug Interface specification.

This enables:

- debugging tools

- breakpoints

- system inspection

1.6 Product Revisions

Processor versions are identified using revision numbers.

Format:

rnpn

Where:

- rn → major revision

- pn → minor revision

Example:

r0p0

Meaning:

- major revision = 0

- minor revision = 0

The manual currently describes the first release r0p0 of the Cortex-M0 processor.

Chapter 3: Programmer’s Model (Cortex-M0)

Chapter 3 explains the programming model of the Cortex-M0 processor. It describes how software interacts with the processor, including registers, instruction set, memory model, and exception handling.

The chapter contains the following sections:

- 3.1 About the Programmer’s Model

- 3.2 Modes of Operation and Execution

- 3.3 Instruction Set Summary

- 3.4 Memory Model

- 3.5 Processor Core Registers Summary

- 3.6 Exceptions

3.1 About the Programmer’s Model

The programmer’s model defines how a programmer writes software for the Cortex-M0 processor. It specifies:

- Processor registers

- Instruction set

- Memory organization

- Exception handling mechanisms

The Cortex-M0 programmer’s model follows the ARMv6-M architecture.

This chapter provides an overview of:

- processor registers

- instruction execution

- memory access mechanisms

- interrupt handling

More detailed descriptions are provided in the ARMv6-M Architecture Reference Manual.

DDI0432C_cortex_m0_r0p0_trm

3.2 Modes of Operation and Execution

Unlike many other ARM processors, the Cortex-M0 operates in a simplified execution model.

Single Privilege Level

The processor does not support separate privileged and unprivileged modes.

This means:

- All software runs in privileged mode

- Programs can access all system resources

This simplifies system design and software development.

Execution State

The processor executes instructions in Thumb state.

Characteristics:

- Instructions are 16-bit or 32-bit Thumb instructions

- Instructions must be halfword aligned

This improves code density and reduces memory requirements.

3.3 Instruction Set Summary

The Cortex-M0 implements the ARMv6-M Thumb instruction set.

This instruction set includes:

- Most 16-bit Thumb instructions

- Some 32-bit Thumb-2 instructions

Examples of supported instructions:

1. Data Processing Instructions

Used for arithmetic and logical operations.

Examples:

- ADD – Addition

- SUB – Subtraction

- MUL – Multiplication

- AND – Logical AND

- ORR – Logical OR

- EOR – Exclusive OR

2. Move Instructions

Used to move data between registers.

Examples:

- MOV – Move register data

- MOVS – Move immediate value

3. Comparison Instructions

Used for comparisons before branching.

Examples:

- CMP – Compare registers

- CMN – Compare negative

4. Shift Instructions

Used to shift bits in registers.

Examples:

- LSL – Logical shift left

- LSR – Logical shift right

- ASR – Arithmetic shift right

- ROR – Rotate right

5. Load Instructions

Used to load data from memory into registers.

Examples:

- LDR – Load word

- LDRB – Load byte

- LDRH – Load halfword

6. Store Instructions

Used to store register values into memory.

Examples:

- STR – Store word

- STRB – Store byte

- STRH – Store halfword

7. Stack Operations

Used for stack manipulation.

Examples:

- PUSH – Push registers to stack

- POP – Pop registers from stack

8. Branch Instructions

Used for program control flow.

Examples:

- B – Branch

- BL – Branch with link

- BX – Branch exchange

9. System Instructions

Used for processor control.

Examples:

- WFI – Wait for interrupt

- WFE – Wait for event

- NOP – No operation

- BKPT – Breakpoint

The cycle count for each instruction depends on the system configuration and memory wait states.

DDI0432C_cortex_m0_r0p0_trm

Binary Compatibility

The Cortex-M0 instruction set is binary compatible with other Cortex-M processors.

This means:

- Programs written for Cortex-M0 can run on other Cortex-M processors.

- Software portability is improved.

However, developers must follow certain rules such as:

- Access NVIC registers using word transfers only.

- Do not modify unused registers.

3.4 Memory Model

The Cortex-M0 uses a structured memory map to organize system memory and peripherals.

The processor includes a bus matrix that manages memory access between:

- Processor core

- Debug Access Port

- Memory system

- NVIC and debug components

Processor accesses always have higher priority than debug accesses.

DDI0432C_cortex_m0_r0p0_trm

Memory Address Regions

The memory map is divided into several regions.

| Address Range | Usage |
| --- | --- |
| 0x00000000 – 0x1FFFFFFF | Code and data memory |
| 0x20000000 – 0x3FFFFFFF | SRAM |
| 0x40000000 – 0x5FFFFFFF | Peripheral devices |
| 0x60000000 – 0x9FFFFFFF | External memory |
| 0xA0000000 – 0xDFFFFFFF | External devices |
| 0xE0000000 – 0xEFFFFFFF | NVIC and system control |
| 0xF0000000 – 0xFFFFFFFF | Vendor-specific devices |

If software tries to execute code from a region that does not allow execution, the processor generates a HardFault exception.

DDI0432C_cortex_m0_r0p0_trm

3.5 Processor Core Registers Summary

The Cortex-M0 contains several 32-bit registers.

General-Purpose Registers

Registers R0–R12 are used for general data operations.

Functions:

- arithmetic operations

- temporary data storage

- parameter passing

Stack Pointer (R13)

Register R13 is used as the stack pointer.

There are two stack pointers:

- MSP (Main Stack Pointer)

- PSP (Process Stack Pointer)

These pointers manage the system stack.

Link Register (R14)

Register R14 is the Link Register (LR).

Purpose:

- Stores return addresses for function calls

- Used during subroutine execution

Program Counter (R15)

Register R15 is the Program Counter (PC).

Functions:

- Holds the address of the next instruction

- Controls program execution flow

Program Status Register (PSR)

The PSR combines three status registers:

- APSR – Application Program Status Register

- IPSR – Interrupt Program Status Register

- EPSR – Execution Program Status Register

These registers store information such as:

- condition flags

- current interrupt number

- processor state

PRIMASK Register

The PRIMASK register controls interrupt masking.

Function:

- Disables all interrupts except NMI

CONTROL Register

The CONTROL register determines:

- which stack pointer is used

- processor operating configuration

3.6 Exceptions

Exceptions are events that interrupt normal program execution.

Examples include:

- interrupts

- faults

- system events

When an exception occurs:

- The processor saves the current state.

- The processor jumps to the exception handler.

- After handling the exception, normal execution resumes.

Interrupt Handling Mechanism

The Cortex-M0 uses an advanced interrupt system.

Features include:

Deterministic Interrupt Response

Interrupt response time is predictable.

Interrupt Late Arrival

If a higher-priority interrupt occurs during handling of a lower-priority interrupt, the processor immediately switches to the higher-priority interrupt.

Tail-Chaining

If another interrupt occurs immediately after finishing one interrupt handler:

- the processor directly switches to the next interrupt handler

- stack operations are minimized

This improves system efficiency.

Restart of Interrupted Instructions

If an interrupt occurs during:

- load multiple instructions

- store multiple instructions

- multiplication operations

The processor abandons the instruction temporarily and restarts it after the interrupt is serviced.

Chapter 4: System Control (Cortex-M0)

Chapter 4 explains the system control registers of the Cortex-M0 processor.
These registers are used to configure and control system-level functions such as interrupts, processor identification, system reset, and timer configuration.

The chapter contains two sections:

- 4.1 About System Control

- 4.2 System Control Register Summary

4.1 About System Control

The System Control Space (SCS) contains registers that control various system-level features of the Cortex-M0 processor.

These registers allow software to:

- configure system behavior

- manage interrupts and exceptions

- control system reset

- configure the SysTick timer

- obtain processor identification information

All system control registers are 32-bit registers.

They are accessed through a specific region of the processor memory map.

Key characteristics:

- Located in System Control Space

- Accessed through word transfers only

- Used by system software and operating systems

If a program tries to access these registers using byte or halfword access, the behavior becomes unpredictable.

DDI0432C_cortex_m0_r0p0_trm

4.2 System Control Register Summary

The Cortex-M0 includes several important system control registers.

These registers help control system-level operations and processor configuration.

Main system control registers include:

- SysTick registers

- CPUID register

- Interrupt control registers

- Configuration control registers

- System handler registers

1. SysTick Registers

The SysTick timer is a 24-bit system timer used for periodic interrupts.

It is commonly used for:

- operating system scheduling

- system time measurement

- periodic event generation

If the processor implementation does not include SysTick, these registers return zero when read, and writes are ignored.

DDI0432C_cortex_m0_r0p0_trm

Important SysTick Registers

SYST_CSR (SysTick Control and Status Register)

Controls the operation of the SysTick timer.

Functions include:

- enabling the timer

- enabling interrupt generation

- selecting the clock source

SYST_RVR (SysTick Reload Value Register)

Stores the value that determines how long the timer counts before generating an interrupt.

SYST_CVR (SysTick Current Value Register)

Stores the current value of the timer counter.

SYST_CALIB (SysTick Calibration Register)

Provides calibration information for the SysTick timer.

This value is implementation-defined and configured by the processor manufacturer.

2. CPUID Register

The CPUID register provides identification information about the processor.

Purpose:

- identifies the processor type

- indicates revision number

- provides architecture information

The reset value for the CPUID register is:

0x410CC200

CPUID Register Fields

The CPUID register contains several fields.

Implementer Field

Bits: [31:24]

Identifies the manufacturer.

Value:

0x41 = ARM

Variant Field

Bits: [23:20]

Indicates the major revision number of the processor.

Example:

0x0

Constant Field

Bits: [19:16]

Indicates the processor architecture.

Value:

0xC → ARMv6-M architecture

Part Number Field

Bits: [15:4]

Identifies the processor model.

Value:

0xC20 → Cortex-M0

Revision Field

Bits: [3:0]

Indicates the minor revision number.

Example:

0x0 → r0p0

This allows software and debugging tools to identify the exact processor version.

DDI0432C_cortex_m0_r0p0_trm

3. Interrupt Control State Register (ICSR)

The ICSR register controls interrupt handling.

Functions include:

- setting pending interrupts

- clearing pending interrupts

- identifying the active interrupt

It is used by the system to manage exception and interrupt processing.

4. Application Interrupt and Reset Control Register (AIRCR)

The AIRCR register is used to control system reset and interrupt priority grouping.

Functions include:

- generating system reset

- configuring interrupt priorities

- controlling system reset behavior

5. Configuration and Control Register (CCR)

The CCR register controls configuration settings of the processor.

Examples of configuration options:

- stack alignment

- unaligned access handling

- system behavior configuration

6. System Handler Priority Registers

These registers define the priority of system exceptions.

Examples include:

- SHPR2 – System Handler Priority Register 2

- SHPR3 – System Handler Priority Register 3

These registers determine the priority levels for system exceptions such as:

- SysTick

- PendSV

- SVCall

7. System Handler Control and State Register (SHCSR)

The SHCSR register provides status information about system exceptions.

Functions include:

- enabling system handlers

- monitoring exception status

- identifying active faults

Important Notes About System Control Registers

- All registers are 32 bits wide.

- They must be accessed using word transfers only.

- Byte or halfword access is unpredictable.

- Some registers may behave differently depending on processor configuration.

Chapter 4: System Control (Cortex-M0)

Chapter 4 explains the system control registers of the Cortex-M0 processor.
These registers are used to configure and control system-level functions such as interrupts, processor identification, system reset, and timer configuration.

The chapter contains two sections:

- 4.1 About System Control

- 4.2 System Control Register Summary

4.1 About System Control

The System Control Space (SCS) contains registers that control various system-level features of the Cortex-M0 processor.

These registers allow software to:

- configure system behavior

- manage interrupts and exceptions

- control system reset

- configure the SysTick timer

- obtain processor identification information

All system control registers are 32-bit registers.

They are accessed through a specific region of the processor memory map.

Key characteristics:

- Located in System Control Space

- Accessed through word transfers only

- Used by system software and operating systems

If a program tries to access these registers using byte or halfword access, the behavior becomes unpredictable.

DDI0432C_cortex_m0_r0p0_trm

4.2 System Control Register Summary

The Cortex-M0 includes several important system control registers.

These registers help control system-level operations and processor configuration.

Main system control registers include:

- SysTick registers

- CPUID register

- Interrupt control registers

- Configuration control registers

- System handler registers

1. SysTick Registers

The SysTick timer is a 24-bit system timer used for periodic interrupts.

It is commonly used for:

- operating system scheduling

- system time measurement

- periodic event generation

If the processor implementation does not include SysTick, these registers return zero when read, and writes are ignored.

DDI0432C_cortex_m0_r0p0_trm

Important SysTick Registers

SYST_CSR (SysTick Control and Status Register)

Controls the operation of the SysTick timer.

Functions include:

- enabling the timer

- enabling interrupt generation

- selecting the clock source

SYST_RVR (SysTick Reload Value Register)

Stores the value that determines how long the timer counts before generating an interrupt.

SYST_CVR (SysTick Current Value Register)

Stores the current value of the timer counter.

SYST_CALIB (SysTick Calibration Register)

Provides calibration information for the SysTick timer.

This value is implementation-defined and configured by the processor manufacturer.

2. CPUID Register

The CPUID register provides identification information about the processor.

Purpose:

- identifies the processor type

- indicates revision number

- provides architecture information

The reset value for the CPUID register is:

0x410CC200

CPUID Register Fields

The CPUID register contains several fields.

Implementer Field

Bits: [31:24]

Identifies the manufacturer.

Value:

0x41 = ARM

Variant Field

Bits: [23:20]

Indicates the major revision number of the processor.

Example:

0x0

Constant Field

Bits: [19:16]

Indicates the processor architecture.

Value:

0xC → ARMv6-M architecture

Part Number Field

Bits: [15:4]

Identifies the processor model.

Value:

0xC20 → Cortex-M0

Revision Field

Bits: [3:0]

Indicates the minor revision number.

Example:

0x0 → r0p0

This allows software and debugging tools to identify the exact processor version.

DDI0432C_cortex_m0_r0p0_trm

3. Interrupt Control State Register (ICSR)

The ICSR register controls interrupt handling.

Functions include:

- setting pending interrupts

- clearing pending interrupts

- identifying the active interrupt

It is used by the system to manage exception and interrupt processing.

4. Application Interrupt and Reset Control Register (AIRCR)

The AIRCR register is used to control system reset and interrupt priority grouping.

Functions include:

- generating system reset

- configuring interrupt priorities

- controlling system reset behavior

5. Configuration and Control Register (CCR)

The CCR register controls configuration settings of the processor.

Examples of configuration options:

- stack alignment

- unaligned access handling

- system behavior configuration

6. System Handler Priority Registers

These registers define the priority of system exceptions.

Examples include:

- SHPR2 – System Handler Priority Register 2

- SHPR3 – System Handler Priority Register 3

These registers determine the priority levels for system exceptions such as:

- SysTick

- PendSV

- SVCall

7. System Handler Control and State Register (SHCSR)

The SHCSR register provides status information about system exceptions.

Functions include:

- enabling system handlers

- monitoring exception status

- identifying active faults

Important Notes About System Control Registers

- All registers are 32 bits wide.

- They must be accessed using word transfers only.

- Byte or halfword access is unpredictable.

- Some registers may behave differently depending on processor configuration.

Chapter 5: Nested Vectored Interrupt Controller (NVIC)

Chapter 5 explains the Nested Vectored Interrupt Controller (NVIC) used in the Cortex-M0 processor.
The NVIC is responsible for handling interrupts and exceptions efficiently.

It provides fast interrupt response, priority management, and low-latency interrupt processing, which is very important in real-time embedded systems.

The chapter contains two sections:

- 5.1 About the NVIC

- 5.2 NVIC Register Summary

5.1 About the NVIC

The Nested Vectored Interrupt Controller (NVIC) manages all interrupts in the Cortex-M0 processor.

External interrupt signals are connected to the NVIC, and the NVIC determines:

- which interrupt should be serviced

- the priority of each interrupt

- when the processor should switch to an interrupt service routine

The NVIC is closely integrated with the processor core, which allows very fast interrupt handling with minimal latency.

DDI0432C_cortex_m0_r0p0_trm

Main Features of NVIC

1. External Interrupt Inputs

The NVIC supports a configurable number of external interrupts.

Possible numbers include:

- 1 interrupt

- 2 interrupts

- 4 interrupts

- 8 interrupts

- 16 interrupts

- 24 interrupts

- 32 interrupts

These interrupts come from external hardware devices such as:

- sensors

- communication interfaces

- timers

- peripheral devices

2. Interrupt Priority Levels

Each interrupt can be assigned one of four priority levels.

Purpose:

- Determines which interrupt should be executed first

- Allows critical tasks to interrupt lower-priority tasks

Higher priority interrupts are serviced before lower priority interrupts.

3. Non-Maskable Interrupt (NMI)

The Cortex-M0 includes a dedicated Non-Maskable Interrupt (NMI).

Characteristics:

- Highest priority interrupt

- Cannot be disabled

- Used for critical system events

Examples include:

- hardware failure

- watchdog timeout

- emergency shutdown conditions

4. Interrupt Trigger Types

The NVIC supports two types of interrupt signals:

Level-Sensitive Interrupts

The interrupt remains active while the signal remains high or low.

Pulse-Sensitive Interrupts

The interrupt is triggered by a short pulse.

This flexibility allows different hardware devices to generate interrupts.

Low-Power Support

The NVIC supports low-power features through optional components.

Wake-up Interrupt Controller (WIC)

Some implementations include a Wake-up Interrupt Controller.

Functions:

- Monitors interrupts while the processor is in sleep mode

- Wakes the processor when an interrupt occurs

- Enables ultra-low-power operation

This is important for battery-powered embedded systems.

DDI0432C_cortex_m0_r0p0_trm

Sleep and Event Instructions

The Cortex-M0 supports instructions that help manage low-power operation.

WFI – Wait For Interrupt

The processor enters sleep mode until an interrupt occurs.

WFE – Wait For Event

The processor enters sleep mode until an event occurs.

SEV – Send Event

Used to wake processors waiting for an event.

Sleep-on-Exit Feature

The processor also supports Sleep-on-Exit.

Function:

- When an interrupt handler finishes, the processor automatically returns to sleep mode instead of normal program execution.

This helps reduce power consumption.

5.2 NVIC Register Summary

The NVIC contains several registers used to control interrupts.

All NVIC registers:

- are 32-bit registers

- must be accessed using word transfers

- do not support byte or half-word access.

DDI0432C_cortex_m0_r0p0_trm

Important NVIC Registers

1. ISER – Interrupt Set-Enable Register

Purpose:

- Enables specific interrupts.

When a bit in this register is set to 1, the corresponding interrupt is enabled.

2. ICER – Interrupt Clear-Enable Register

Purpose:

- Disables specific interrupts.

Setting a bit in this register disables the corresponding interrupt.

3. ISPR – Interrupt Set-Pending Register

Purpose:

- Manually sets an interrupt as pending.

This can be used by software to trigger an interrupt.

4. ICPR – Interrupt Clear-Pending Register

Purpose:

- Clears the pending state of an interrupt.

Used to cancel a pending interrupt request.

5. IPR Registers – Interrupt Priority Registers

Registers:

- IPR0

- IPR1

- IPR2

- IPR3

- IPR4

- IPR5

- IPR6

- IPR7

Purpose:

- Set the priority level of interrupts.

These registers allow software to control the order in which interrupts are handled.

Advantages of NVIC

The NVIC provides several benefits:

1. Fast Interrupt Response

The NVIC is integrated with the processor, allowing quick interrupt handling.

2. Priority-Based Processing

Interrupts are serviced based on priority levels.

3. Efficient Interrupt Handling

The NVIC supports advanced features such as:

- tail-chaining

- late arrival

These features reduce interrupt handling time.

4. Low Power Support

Combined with WFI and WFE instructions, the NVIC helps reduce power consumption.