Chapter 1: Introduction to the I2C-Bus

- Background: The Inter-IC (I2C) bus was developed by Philips Semiconductors (now NXP) to provide a simple, efficient, bidirectional 2-wire bus for controlling integrated circuits,.

- Industry Standard: It is a de facto world standard implemented in over 1,000 different ICs manufactured by more than 50 companies.

- Architecture Base: The I2C protocol is highly versatile and serves as the foundation for other control architectures like SMBus, PMBus, IPMI, DDC, and ATCA.

- Evolution (I3C): For applications requiring even faster speeds and lower power, the MIPI Alliance introduced I3C in 2017, which offers backward compatibility with I2C.

Chapter 2: I2C-Bus Features & Benefits

- Core Technical Features:

  - Requires only two lines: SDA (Serial Data) and SCL (Serial Clock).

  - Each connected device is software-addressable by a unique address.

  - It is a true multi-controller bus featuring collision detection and arbitration to prevent data corruption when multiple controllers initiate transfers simultaneously.

  - Features on-chip filtering to reject spikes on the data line, ensuring data integrity.

  - Capacity is limited only by maximum bus capacitance,.

- Supported Speed Modes:

  - Standard-mode (Sm): Up to 100 kbit/s.

  - Fast-mode (Fm): Up to 400 kbit/s.

  - Fast-mode Plus (Fm+): Up to 1 Mbit/s.

  - High-speed mode (Hs): Up to 3.4 Mbit/s.

  - Ultra Fast-mode (UFm): Up to 5 Mbit/s (Unidirectional).

- Key Benefits:

  - For Designers: Rapid design from block diagram to schematic without needing custom bus interfaces, completely software-defined systems, simple fault diagnosis, high noise immunity, and low current consumption,,,.

  - For Manufacturers: Reduces PCB tracks and IC pin counts (making smaller and cheaper PCBs), eliminates the need for glue logic, and allows for rapid assembly line testing.

Chapter 3: The I2C-Bus Protocol Mechanics

1. Bus Terminology

- Transmitter/Receiver: The device sending data to the bus vs. the device receiving data.

- Controller: The device that initiates the transfer, generates clock signals, and terminates the transfer (replaces legacy term "Master"),,.

- Target: The device addressed by a controller (replaces legacy term "Slave"),,.

2. Physical Layer & Data Validity

- Hardware Setup: Standard I2C requires open-drain or open-collector output stages to perform a "wired-AND" function. Both lines must be connected to a positive supply voltage via pull-up resistors.

- Data Validity: Data on the SDA line must remain stable while the SCL clock is HIGH. Data states can only change when SCL is LOW.

3. Framing a Transfer

- START (S) Condition: Initiated by the controller pulling SDA HIGH to LOW while SCL is HIGH.

- Byte Format: Data is transferred 8 bits at a time, Most Significant Bit (MSB) first.

- ACK / NACK: After every 8-bit byte, a 9th clock pulse is generated. The receiver pulls SDA LOW to acknowledge (ACK) successful receipt. If SDA is left HIGH, it registers as a Not Acknowledge (NACK),.

- STOP (P) Condition: The controller pulls SDA LOW to HIGH while SCL is HIGH, freeing the bus. A "Repeated START" (Sr) can be used instead of a STOP if the controller wants to keep the bus busy for subsequent transfers.

4. Multi-Controller Operations

- Clock Synchronization: If multiple controllers start simultaneously, the wired-AND connection synchronizes their clocks. The SCL LOW period is dictated by the controller with the longest LOW period, while the HIGH period is dictated by the controller with the shortest HIGH period,,.

- Arbitration: If two controllers transmit data simultaneously, the first controller to try sending a HIGH (1) while detecting a LOW (0) on the SDA line loses arbitration and must switch off its output.

5. Clock Stretching & Addressing

- Clock Stretching: A target device can hold the SCL line LOW to force the controller into a wait state if it needs time to process data or service an interrupt,.

- Addressing Format: Following the START condition, a 7-bit target address is transmitted, followed by an 8th Read/Write bit (0 = Write, 1 = Read). 10-bit addressing is also supported, relying on the reserved prefix 1111 0XX.

- Special Addresses:

  - General Call (0000 000 0): Addresses every device on the bus simultaneously,.

  - Software Reset: Sending 06h after a general call resets devices to take in their programmable addresses.

  - Device ID: A 24-bit read-only word that identifies the manufacturer (12 bits), part ID (9 bits), and die revision (3 bits).

6. Ultra Fast-mode (UFm) Protocol Variations

- Unidirectional: Operates from DC to 5 MHz transmitting data strictly in one direction (Controller to Target) using push-pull drivers.

- No Multi-Controller or Clock Stretching: Only one controller is allowed, and targets cannot alter the clock signal,.

- No Acknowledgments: Because it is unidirectional, targets cannot pull the data line low. The 9th bit (ACK cycle) is preserved for structural compatibility, but the controller simply drives the line HIGH instead of reading an ACK